⚡ ELECTRICAL CIRCUITS LAB REPORT ⚡

Hybrid 1-Way & 2-Way Lighting Control Circuit

📋 PROJECT INFORMATION

| Experiment | Hybrid Lighting Control Circuit |
| --- | --- |

| Circuit Type | Series & Parallel (Hybrid) |
| --- | --- |
| Switch Types | 1-Way Switch ¦ 2-Way Switch |

| Components | 2 Lamps, 1 One-Way Switch, 2 Two-Way Switches, AC Power Source |
| --- | --- |

🎯 EXPERIMENT OBJECTIVE

To design and implement a hybrid lighting control circuit that demonstrates both 1-way and 2-way switch configurations using a combination of series and parallel wiring techniques, replicating real-world residential lighting principles.

🔌 CIRCUIT DESCRIPTION

⚡ 1-WAY SWITCH CONFIGURATION The first section features a 1-way switch connected in series with a single lamp. Current flows from the AC power source, through the switch, and then to the lamp. This configuration allows the lamp to be controlled from a single location — typical of standard bedrooms or bathrooms.

🔄 2-WAY SWITCH CONFIGURATION The second section employs a 2-way switching system using a traveler/parallel configuration to control the second lamp. This enables the lamp to be turned ON or OFF from either switch regardless of the other's position — ideal for hallways and staircases where multiple control points are required.

🗺️ CIRCUIT LAYOUT & WIRING COLOUR CODE

| 🔴 RED WIRES | Traveler connections between the two 2-way switches |
| --- | --- |

| ⬛ BLACK WIRES | Live (main) and return (neutral) paths |
| --- | --- |

📊 SIMULATION RESULTS

CIRCUIT BEHAVIOUR UNDER DIFFERENT SWITCHING STATES

| Switch 1 (1-Way) | Switch 2 (2-Way) | Switch 3 (2-Way) | Result |
| --- | --- | --- | --- |
| ON | UP | UP | Both ON ✅ |
| OFF | UP | UP | Lamp 1 OFF ❌ |
| ON | UP | DOWN | Lamp 2 ON ✅ |

Experimental Result

Stimulation Result

🔍 OBSERVATIONS & INSIGHTS

| ▶ | Lights occasionally exhibited reduced brightness depending on switching states — attributed to internal resistance and unequal current distribution. |
| --- | --- |
| ▶ | Voltage drops were observed in shared segments of the hybrid circuit, highlighting the impact of circuit design on performance. |
| ▶ | The 2-way switch traveler system successfully allowed the lamp to be toggled from either switch location independently. |
| ▶ | Precise wiring was confirmed to be critical — minor wiring errors caused inconsistent lamp behaviour during initial testing. |
| ▶ | The 1-way switch section performed reliably, with the lamp responding immediately and consistently to switch toggling. |

✅ CHOSEN SOLUTION — HYBRID CIRCUIT

WHY THE HYBRID CIRCUIT WAS SELECTED

| ▶ | Allows independent control of each lamp using different switch configurations. |
| --- | --- |
| ▶ | Ensures continued functionality even if one lamp or switch section becomes non-operational. |
| ▶ | Provides a clear educational illustration of both series (1-way) and parallel (2-way) connections. |
| ▶ | Incorporates commonly used switch types found in real residential wiring applications. |
| ▶ | Meets all technical and safety criteria while serving as a valuable teaching tool. |

📝 CONCLUSION

This experiment successfully demonstrated the fundamental principles of residential lighting control. The hybrid circuit combining a 1-way switch in series with a single lamp, and a 2-way switch traveler system for the second lamp, effectively illustrated how both simple and complex control systems coexist in unified circuits. Key principles covered: switch logic, circuit configuration, voltage drop behaviour, load sharing, and practical effects of electrical design — all confirmed through hands-on testing and observation.

⚡ Experiment Completed Successfully ⚡