ENGINEERING WORKSHOP Experiment Report OPEN ENDED LAB

Hybrid Series–Parallel Lighting Control Circuit

| ASPECT | DETAIL |
| --- | --- |
| Submitted by | Zainab Bilal (24L-6127) |
| Submitted to | Ms Aqbari Yaqoob |
| Department | Electrical Engineering |
| Institution | National University of Computer and Emerging Sciences, Lahore |
| Date | 23 April 2025 |

# Table of Contents

Table of Contents	1

1.  Introduction	1

2.  Problem Analysis	1

3.  Design Requirements	1

4.  Feasibility Analysis	1

5.  Possible Solutions	1

6.  Preliminary Design	1

7.  Design Description	1

8.  Software Simulation	1

9.  Experimental Results	1

10.  Performance Analysis	1

11.  Future Scope	1

12.  Social and Cultural Implications	1

13.  Conclusion	1

14.  References	1

# 1.  Introduction

Lighting control plays a vital role in contemporary electrical systems. This project focuses on creating a circuit that incorporates both series and parallel configurations, utilizing 1-way and 2-way switches to operate two lamps. It serves as a practical demonstration of key principles used in residential and commercial electrical wiring.

Project Snapshot ▸ Goal: Control two lamps using a combined series + parallel hybrid circuit ▸ Switch types: One 1-way switch and one 2-way (traveler) switching system ▸ Relevance: Mirrors real wiring found in hallways, staircases, and single-room lighting

# 2.  Problem Analysis

## Task Overview

The goal of this project is to design and implement a lighting circuit that utilizes both series and parallel connections to control two light bulbs. The circuit features one 1-way switch and one 2-way switch, offering a hands-on demonstration of common wiring techniques typically used in residential and commercial lighting setups, such as in homes or hallway systems.

## Problem Statement

This project involves designing a circuit capable of effectively controlling two lamps through a combination of series and parallel wiring, using both a 1-way and a 2-way switch. The challenge lies in configuring the circuit so that each lamp can be operated independently while complying with standard wiring conventions.

## Key Requirements

- Understanding the operation and wiring of 1-way and 2-way switches.

- Integrating both switch types into a unified, functional circuit.

- Ensuring proper switching logic for correct lamp operation.

- Maintaining safety by correctly handling live and neutral connections.

## Proposed Solution

| Power Supply | 1-Way Switch ➜ Lamp 1 | 2-Way Switches ➜ Lamp 2 | Series + Parallel Network |
| --- | --- | --- | --- |

- The first lamp is controlled directly by a 1-way switch.

- The second lamp is managed by a 2-way switch, which allows control through an alternative circuit path.

- The live wire from the power supply is routed through both switches to distribute power.

- A combination of series and parallel wiring connects the lamps, offering both control flexibility and logical interdependence between switches and loads.

This Design Ensures ▸ Independent operation of each lamp via its corresponding switch ▸ Clear demonstration of lighting control logic and wiring practices ▸ A simple yet effective layout — ideal for educational use and practical illustration

# 3.  Design Requirements

## Circuit Design Specifications

To ensure the circuit functions correctly and operates safely, the design must adhere to the following specifications:

| ASPECT | DETAIL |
| --- | --- |
| Power Supply | A standard AC voltage source to energize the circuit. |
| Switches | One 1-way switch and one 2-way switch to manage lamp control. |
| Wiring | Colour-coded wires — Red for Live, Black for Neutral — for safety and easy identification. |
| Lamps (Output) | Two light bulbs providing visual confirmation of switch operation; each lamp responds precisely to its designated switch. |

## Functionality Constraints

- The first lamp must be directly controlled by the 1-way switch.

- The second lamp must be operated via the 2-way switch configuration.

- The circuit must include both series and parallel wiring elements to simulate realistic electrical layouts.

## Safety Constraints

Safety First ▸ All live and neutral wires must be well-insulated and securely fastened to minimise shock or accidental disconnection risk. ▸ The design must be resilient against short circuits and overheating, ensuring long-term safety during operation and demonstration.

## Physical Setup Requirements

- All components must be firmly mounted onto a testing or demonstration board for structural integrity and safety.

- Switches and lamps should be clearly labelled and easily reachable, facilitating user interaction, observation, and instructional demonstrations.

# 4.  Feasibility Analysis

This project is highly practical and achievable, both in terms of time and cost. It utilizes common components and follows a simple assembly process, making it ideal for academic lab settings.

| 1.5–2 hrs LAB SESSION TIME | Low COMPONENT COST | Minimal TROUBLESHOOTING | Reusable KEY COMPONENTS |
| --- | --- | --- | --- |

## Time Management

- The entire project can be completed within a standard laboratory session, typically spanning 1.5 to 2 hours.

- This duration includes planning the circuit layout, assembling components, testing, and verifying results.

- Thanks to the simplicity of the design, troubleshooting requirements are minimal, contributing to a smooth and time-efficient process.

## Cost Management

- All required components — switches, wiring, lamp holders, and bulbs — are low-cost and widely available in basic electrical labs.

- No specialised or expensive equipment is necessary for implementation.

- Many components are reusable, helping to minimise ongoing costs for future iterations.

## Resource Constraints

- The project only requires basic electrical tools: wire cutter, screwdriver, and tester.

- No digital simulations or external modules are needed, eliminating the need for additional software or advanced hardware.

Feasibility Verdict ▸ The project satisfies all key feasibility criteria — cost-effective, time-efficient, and resource-light — making it well-suited for a standard lab environment.

# 5.  Possible Solutions

To effectively control two lamps using a combination of a 1-way and a 2-way switch, two possible circuit configurations were evaluated:

## Option 1 — Series-Only Circuit

Both lamps are connected in a single series path and controlled either by a single switch or in sequence.

## Option 2 — Hybrid Circuit (Series + Parallel)

This setup combines series and parallel wiring to allow each lamp to be independently controlled — one via a 1-way switch and the other through a 2-way switch.

| Criteria | Series-Only Circuit | Hybrid Circuit ✓ |
| --- | --- | --- |
| Wiring complexity | Low | Moderate |
| Independent lamp control | No | Yes |
| Fault tolerance | Poor — one failure stops all | Good — isolated paths |
| Real-world relevance | Limited | High (hallway / staircase systems) |
| Educational value | Basic | Demonstrates series + parallel + switching logic |

Chosen Solution: Hybrid Circuit ✓ ▸ Allows independent control of each lamp ▸ Ensures continued functionality even if one lamp is non-operational ▸ Provides an educational illustration of both series and parallel connections with common switch types

# 6.  Preliminary Design

The preliminary layout maps the power path from the AC source through both switching stages to the two lamps, combining a direct series branch with a parallel traveler branch:

| AC Supply (Live/Neutral) | 1-Way Switch | Lamp 1 (Series) |
| --- | --- | --- |

Source   ➜   Switch   ➜   Load

| AC Supply (Live/Neutral) | 2-Way Switch A | Traveler Wires | 2-Way Switch B | Lamp 2 (Parallel) |
| --- | --- | --- | --- | --- |

Source   ➜   Switch A   ➜   Travelers   ➜   Switch B   ➜   Load

# 7.  Design Description

## Experiment Summary

In this experiment, we designed and implemented a lighting control circuit that demonstrates the use of both 1-way and 2-way switches, incorporating a combination of series and parallel wiring techniques.

## 1-Way Switch Configuration

The first section of the circuit features a 1-way switch connected in series with a single lamp. Current flows from the AC power source, through the switch, and then to the lamp — allowing the lamp to be controlled from a single location, similar to standard lighting setups in rooms like bedrooms or bathrooms.

## 2-Way Switch Configuration

The second section employs a 2-way switching system to control the second lamp using two 2-way switches. This setup uses a parallel configuration — specifically a traveler system — enabling the lamp to be turned on or off from either switch, regardless of the other's position. This is typical in hallway or staircase lighting where multiple control points are needed.

Observations & Insights ▸ Lights occasionally showed reduced brightness depending on switching states and current paths — attributed to internal resistance and unequal current distribution in shared circuit segments. ▸ This highlighted how design choices affect voltage drops and load sharing, reinforcing the importance of precise wiring in real-world applications.

## Circuit Layout Details

- The red wires represent the traveler connections between the two 2-way switches.

- The black wires are used for the live (main) and return (neutral) paths.

- This wiring scheme illustrates how simple (1-way) and complex (2-way) control systems coexist within one unified circuit.

# 8.  Software Simulation

No digital simulation software was required for this build — the hybrid circuit was validated directly through physical assembly and hands-on testing, as outlined in the Feasibility Analysis.

# 9.  Experimental Results

The physical circuit was assembled and energised, with both lamps responding correctly to their respective switches:

Figure 1: Hybrid series–parallel circuit in operation — both lamps illuminated

# 10.  Performance Analysis

The circuit was successfully constructed and tested using both AC and DC power sources. As intended, Lamp 1 was controlled via a 1-way switch in a series configuration, while Lamp 2 was operated through a 2-way switch system in a parallel configuration.

| 100% SWITCH ACCURACY | 0 SHORT CIRCUITS | 2 CONTROL POINTS (LAMP 2) | Stable OPERATION |
| --- | --- | --- | --- |

## Key Observations

- Both lamps responded accurately and consistently to their designated switches.

- The 2-way switch system functioned correctly, allowing Lamp 2 to be controlled from two separate locations — confirming the effectiveness of the traveler configuration.

- The circuit performed without any issues such as short circuits, flickering, or wiring faults during testing.

## Performance Summary

- Switching behaviour was smooth and reliable, with no lag or instability.

- The setup demonstrated safe and efficient operation, meeting all design and safety standards.

- The project successfully integrated both series and parallel switching mechanisms, reflecting real-world residential and commercial lighting control systems.

# 11.  Future Scope

This foundational circuit can be extended toward more advanced and intelligent lighting control systems for future iterations.

- Integration with smart relays or microcontrollers (e.g., Arduino/ESP32) for app- or voice-controlled switching.

- Addition of dimming circuitry to address the brightness variation observed during testing.

- Expansion to multi-lamp, multi-zone hallway or staircase systems using additional 2-way/intermediate switches.

- Incorporation of energy-monitoring sensors to track power consumption per lamp.

# 12.  Social and Cultural Implications

Why It Matters ▸ Reliable, well-understood lighting control improves safety and accessibility in homes, schools, and public buildings. ▸ Traveler (2-way) switching is widely used in South Asian residential construction, making this an immediately transferable, locally relevant skill. ▸ Hands-on wiring literacy supports safer DIY practices and reduces electrical-fault risks in everyday households.

# 13.  Conclusion

The experiment successfully met all of its objectives, demonstrating effective control of two lamps through a hybrid series-parallel circuit using both 1-way and 2-way switches. This setup not only illustrated key concepts in switch logic and wiring configurations but also provided valuable hands-on experience in practical circuit design. The project served as a foundational exercise, offering insights that can be applied to more advanced systems, including automated lighting and smart control applications in future projects.

# 14.  References

- Engineering Workshop Lab Manual — Department of Electrical Engineering, FAST-NUCES Lahore.

- Standard residential wiring practice guidelines for 1-way and 2-way (traveler) switching systems.